PERMIT TO WORK — EXCAVATION & GROUND WORKS CDM 2015 · Service Strike Prevention · Ground Stability

| Valid From: | Valid Until: | Created By: | Permit #: |
| --- | --- | --- | --- |

| High-Risk Task(s) Covered by this Permit — Check All That Apply |  |  |  |
| --- | --- | --- | --- |
| ☐ Hot Work | ☐ Work at Height | ☐ Plant Isolation | ☐ Other (specify): |
| ☐ Confined Space | ☐ Live Electricity | ☐ Excavation |  |

| General Information |  |
| --- | --- |
| Project Name: |  |
| Client: |  |
| Contractor / Department: |  |
| Supervisor-in-Charge: |  |
| Description of Work: | Excavation or ground disturbance where underground services, ground collapse, or flooding present foreseeable risk. Services located and marked before any intrusive work commences. |
| Applicable Standards: | CDM Regulations 2015; HSG47 (Avoiding Danger from Underground Services); HS(G)185; BS 6031:2009 (Earthworks) |

| Pre-Job Conditions & Requirements |  |
| --- | --- |
| Equipment / Tools Used: | Cable/pipe locator (CAT & Genny), current service drawings, trial holes, hand tools for service zone, trench support (trench box / shoring / battered sides), banksman, pumping equipment (if groundwater risk), edge protection barriers, ladder for access and egress |
| PPE Required: | Hard hat (EN 397); high-visibility vest; safety footwear; gloves; eye protection; RPE if contaminated ground suspected; hearing protection if mechanical excavation |
| Additional Permits Required: | COSHH Assessment if contaminated ground; Hot Works Permit if cutting services |

| Identified Hazards |  |
| --- | --- |
| ☐ Striking underground services — gas, electricity, water, or telecoms | ☐ Ground collapse / trench cave-in causing burial or crush |
| ☐ Flooding or sudden ingress of groundwater | ☐ Contaminated ground — hazardous gases, asbestos, or chemicals |
| ☐ Falls of persons or plant into the excavation | ☐ Plant / vehicle overrun or toppling into the excavation edge |

| Safety Controls & Precautions |  |
| --- | --- |
| ☐ Services located using CAT & Genny sweep and current service drawings | ☐ Hand dig / vacuum excavation used within 500 mm of all marked services |
| ☐ Competent person inspection completed and recorded before each shift | ☐ Trench support in place — trench box / shoring / battered to safe angle |
| ☐ Edge protection barriers set ≥ 1 m from excavation; spoil set back ≥ 1 m | ☐ Pumping equipment available; groundwater level monitored continuously |
| ☐ Access ladder(s) positioned — within 9 m of all workers at all times |  |

| Emergency Procedure |  |
| --- | --- |
| In the event of emergency: | In the event of a service strike: stop all work immediately and clear the area. Do not use naked flames or electrical equipment near a gas escape. Call 999 and the relevant utility emergency line. In the event of collapse: call 999 immediately — do not enter to attempt rescue. Notify the permit authoriser. Preserve the scene for investigation. |

| Pre-Authorisation Checks — Excavation |  |
| --- | --- |
| ☐ Service drawings reviewed — current revision confirmed | ☐ CAT & Genny sweep complete — all services marked on ground surface |
| ☐ Hand dig zone clearly identified within 500 mm of all marked services | ☐ Competent person inspection complete and recorded in writing |
| ☐ Trench support / battered sides confirmed adequate before any entry | ☐ Access ladder(s) in position, secured, and extending 1 m above ground level |

| Permit Request |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Role | Name | Signature | Date |
| Requested By: |  |  |  |  |

| Permit Approvals |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Role | Name | Signature | Date |
| Approved by: | Site Safety Manager |  |  |  |
|  | Area Supervisor |  |  |  |

| Worker Log & Sign On / Off |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name (First & Last) | Designation | Certs / Accreditations | Sign-On Date | Signature | Sign-Off Date | Signature |

| Permit Extension |  |  |  |  |
| --- | --- | --- | --- | --- |
| Valid From | Valid Until | Approved By | Signature | Date |

| Permit Withdrawal / Close-Out |  |
| --- | --- |
| Work Status: | ☐ Completed ☐ Suspended ☐ Cancelled / Withdrawn |
| Inspected By: |  |
| Signature: |  |
| Approved By: |  |
| Signature: |  |

Complete in full before work commences. Retain on site for duration. Keep completed permits ≥ 3 years.  © Hyper Mode Ltd / PermitDesk — permitdesk.io